Monday

Flip the switch to see if it takes away the spark from the spark plug.

2

Steps Taken to Reach Objective:

Objective (s):

Fix kill switches both in dash and exterior to make sure we can shut off the car when needed.

List issues / Complications:

1

3

4

Look at wiring on the other kill switch to see if we are wired similarly.

Check spark plugs to make sure we first have spark.

Flip the two wires around and take the negative and put it in the middle slot.

Tuesday

Put the battery back in the front of the car and reattach wires and its place holder.

2

Steps Taken to Reach Objective:

Objective (s):

Check the car and find anything that may need fixed, tightened, or worked on.

List issues / Complications:

1

3

4

Check for leaks from the engine and rear end.

We thought the rear end was leaking but we just over filled it.

We added oil to the engine and rear end.

Tighten up loose nuts and bolts so nothing falls off or breaks.

Wednesday

Make sure the two sprockets that drive the chain are vertically the same.

2

Steps Taken to Reach Objective:

Objective (s):

Fix chain and cvt.

List issues / Complications:

1

3

4

Take outside and test

Chain fell off during test.

Remove chain and put on cvt belt.

Adjust height of the sprockets and bring them up so they don’t hit the ground and hopefully stay on.

Thursday

Make sure they were level so we knew they were vertically similar and parallel with the ground.

2

Steps Taken to Reach Objective:

Objective (s):

Continue working on the drive chain.

List issues / Complications:

1

3

4

Check height so it would not hit the ground and fall off.

Adjust the cvt axle because it was crooked thus making the sprocket and chain crooked so it would fall off.

Go outside and test the new setup.

Friday

Press in the new lug bolts

2

Steps Taken to Reach Objective:

Objective (s):

Make new chain tenser. Bleed brakes. (Formula car)

List issues / Complications:

1

3

4

Attach the new spindles so we can bleed the brakes.

Drill holes and bolt in the new piece for the chain.

Bleed the brakes.